How it works

- Radio waves are electromagnetic radiation, like light, but with a lower frequency. All electromagnetic radiation travels at the speed of light, 3.0 x 108 m/s. Radio waves can travel short or long distances depending on the electrical power of the radio transmitter. The micro:bit’s can travel about 70 meters in clear space. Look on the back of your micro:bit card, in the upper left corner. Can you find the radio’s gold antenna? This is where the radio waves go in and out of the card.

- The micro:bit radio emits and receives radio waves with frequencies ranging from 2402 - 2486 megahertz; this range is called the radio’s spectrum, and the micro: bit’s is divided into 1 MHz wide bands called channels. There are 84 different radio channels, ranging from 0 to 83, on the micro:bit. Two or more micro:bits must share the same channel to communicate.

- Text messages are added to radio waves in digital modulation. Text messages are inserted into a packet, including additional information needed for routing and error checking.

- In addition to the micro:bit radio channels, there is also a software group. The group number is part of the message packet used to route the data, similar to TCP/IP packets used on The Internet. The group is one byte of the packet and ranges from 0 – 255.

- For two radios to communicate, they must share the same channel and group.

- When a text message in readable characters is sent over the radio, it is called cleartext. It is vulnerable to eavesdropping by an unseen hacker listening on the same radio channel and group. This type of hacking is called a “man-in-the-middle attack.”

What will you do?

- Organize your team:

  - Work in a team with at least two others, each with an Nspire CX II calculator and micro:bit.

  - Your teacher will assign your team a radio channel number. Do not change the group number.

  - Each group member chooses a sender, receiver, or hacker role.

- Send a text message:

  - The receiver

    - will advance to ‘student_receiver.py,’ change the channel to your assigned number, and run the program before the sender has run theirs.

  - The sender

    - will advance to the ‘student_sender.py,’ edit the message string, change the channel to your assigned number, and then run your program after the receiver and hacker have started theirs.

  - The hacker

    - will advance to ‘student_hacker.py,’ change the channel to your assigned number, and run the program before the sender has run theirs.

  - After your team runs the activity, the sender should change their program to a different channel number (0-83) and also change their message. The sender should whisper the new channel to the receiver, who should then change their program to the same channel number. Don’t tell the hacker; keep it private! Then, rerun the activity. Does the hacker get the new message? Can you explain why?

Code it

| Sender role | Receiver role | Hacker role |
| --- | --- | --- |

Go further

- Try a different role in your group.

- Include another group of students and make a large group text.

- Try the activity with the same channel number but a different group number.

Check your understanding

- The receiver must be listening before the sender transmits the message.

- A radio message can be sent on any combination of the micro:bit’s 84 radio channels or 256 radio groups.

- For micro:bits to communicate, they must be on the same channel and group.

- Messages sent in cleartext over a known channel and group can be hacked.

- Using a secret channel or group can help to prevent hacking.

Help

- Check that everyone in the group is on the same channel and group numbers.

- Ensure the receiver and hacker run their programs and wait before the sender transmits the message.